Mobile rack

- Product Name : Removable Bookcase

- Scope of application  : This specification applies to  mobile rack-making  light installation works installed at  the International Vaccine Research Institute.

- Mobile rack format and specification

| Type | SIZE(mm) |  | Comment |
| --- | --- | --- | --- |
|  | Single Fixation / Single Movement | Doubles Fixation / Doubles Movement |  |
| Normal | 1130 * 400/500 * 2275 | 1130 * 500/650/720 * 2275 |  |
| Connected | 970 * 400/500 * 2275 | 970 * 500/650/720 * 2275 |  |

- Double Fixation: 1ea (3-line, 6-shelf)

- Single Movement: 2ea (5-line, 6-shelf)

- Double Movement: 6ea (5-line, 6-shelf)

- Key Components

| Key Components | Contents | Comment |
| --- | --- | --- |
| Base | Drive part and load support part including drive wheels |  |
| Faceplate | Handle and front panel for mobile rack operation |  |
| Rail | Rail part for smooth movement of the drive caster |  |
| Body | Pillars, line segments, etc. loading space formation part |  |

- form

  - The shape and  dimensions of the mobile shelves  shall be  made according to  the  materials and specifications of  each site,  and the  tolerance of the finished product  shall be ±2.0 mm.

  - The specifications of the mobile shelves are determined by  their width, height, and depth.

  - The mobile shelves are constructed using  a detachable side cover and a special connection structure of rollers.

  - Each rack should have a door that can be locked using a key or password.

  - All mobile racks should have some waterproof functionality for document protection.

  - Every wheel and shelf must be used to withstand a load of weight that mobile rack contains with full conditions.

  - In   order to be  able to run smoothly with  little force during the user's operation, the internal gears must be manufactured with a large reduction ratio.

  - For the safety of the user, a stopper device is  placed on the top of the  control panel  to prevent the  mobile shelves from being operated by others.

  - The side that rubs against the caster must be free  of  wear and tear even with long periods of friction.

- Features and Performance

  - In the same production, the parts of the  product and the  parts  of the constituent materials must be  interchangeable.

  - It  should be made easy to assemble/dismantle and manufactured so that it is possible to increase or decrease the line in the future.

  - The  singular adjustment of the shelf must be  free to maximize the storage efficiency.

  - Between the shelves and the shelves,  a  shock buffer rubber should be attached  to absorb  the impact.

  - The body must not have torsion, deformation,  cracks and must be of perfect quality  for use.

  - Products are delivered in individual packaging and must not be  damaged  or damaged during transportation.

- Processing

  - All materials used  must be free of  deformation,  cracking, and defects in the joints  .

  - Areas where the  surface is widely exposed,  such as the side,  back plate,  and shelf, are  made by inserting at least 1 copy of the reinforcement.

  - The human body and the parts  that come into contact with the article should not have sharp protrusions or edges.

- Surface treatment and painting

  - The paint surface  shall  be free of irregularities,  blotches,  peeling, and uniform  thickness, gloss, and  hue of the paint.

  - The coating uses  a durable paint that  is equal to or greater than the coating prescribed for epoxy polyester powder coatings.

  - All seals are degreased  and  coated pre-treated to completely remove impurities and then paint.