COMMODITY DESCRIPTION

| HSK NO. | ITEM NO. | DESCRIPTION | UNIT | QUANTITY |
| --- | --- | --- | --- | --- |
|  |  | Liquid Nitrogen Storage Tank |  | Total 12 |

- Features

- The LN2 tank refrigerates and stores samples at extremely low temperatures(-196℃) using liquid nitrogen as a refrigerant.

- The chambers in which the samples are stored have rotating compartments, which allow samples to be stored separately.

- The temperature sensors are installed to display the upper and lower temperature inside the freezer.

- The temperature and liquid nitrogen levels can be automatically adjusted, enabling stable sample management.

- It is equipped with a folding foot ladder for convenient input and output of samples, which helps users work easily and safely.

- A convenient top working table and a shelf for sample operations are available.

- It utilizes advanced vacuum insulation to minimize the evaporation of liquid nitrogen and reduce operating costs.

- Various alarm systems are available, including low/high liquid level, low/high temperature and LN2 supply alarm.

- Over the past five years, products from companies that have supplied equipment to facilities such as Biobanks or Biorepository centers, or have experience in connecting to tank lorries.

- Specifications

- Temperature Range: -196℃

- Vial Capacity: 18,000 vials or more

- Electrical data: 220~230V, 50/60Hz

- Weight Empty: about 330 kg

- Weight Full: 700 kg or less

- Remarks

- The installation and operation should be carried out by the bidder/supplier responsibility at end-user's site.

- IQ and OQ must include.

- More than two years warranty service should be provided by supplier after the performance test.

- 위 규격서에서 명시한 제품과 동등 이상의 다른 제품 납품이 가능한 업체는 입찰에 참가할 수 있음.