COMMODITY DESCRIPTION

| HSK NO. | ITEM NO. | DESCRIPTION | UNIT | QUANTITY |
| --- | --- | --- | --- | --- |
|  |  | -80℃ Ultra-Low Temperature Freezer (Two stage cascade refrigeration System, 이원냉각) |  | Total 15 |

Ⅰ. Two stage cascade refrigeration System – 15 SETS

- Features

- Minimize the operational stress of the system to enhance its exceptional reliability and ensure sample protection.

- The inner door prevents loss by efficiently sealing cold air through an effective mechanism.

- Through the emission of minimal heat into the environment, it optimizes energy efficiency by reducing the workload on cooling, heating, and air conditioning systems.

- Samples remain securely protected within the cabinet, with temperature differentials maintained below 5℃.

- It quickly restores the internal temperature upon opening the door, ensuring operational efficiency.

- Operating quietly with noise emission levels below 60 dBA, the freezer maintains a stable environment in hallways.

- It provides systematic storage methods utilizing standardized storage accessories such as racks, trays, and boxes and should be included in the estimate.

- Providing access to crucial data and equipment alerts, including equipment and ambient temperatures, alarms, and warning notifications.

- Featuring ergonomic handle, eye-level display, and strategically positioned shelves, it enables convenient and comfortable access.

- An option is available to designate individuals with access to the samples and monitor who opens the freezer door, as well as when and for how long it was open, ensuring strict security and safe storage.

- The freezer should be capable of accommodating the installation of CO2 and LN2 backup systems at a later.

- Over the past five years, there has been a track record of supplying equipment with data transmission systems.

B. Specifications

- Temperature Range: -60℃to -90℃

- Vial Capacity: 40,000 vials or more (The quantity of vials based on a 9*9 box)

- Weight (excluding rack): 450 kg or less

- Cabinet type/ Door type: Upright/ Single

- Electrical data: 110V, 220V 60Hz, 230V 50Hz

C. Remarks

- The installation and operation should be carried out by the bidder/supplier responsibility at end-user's site.

- IQ and OQ must include.

- More than two years warranty service should be provided by supplier after the performance test.

- 위 규격서에서 명시한 제품과 동등 이상의 다른 제품 납품이 가능한 업체는 입찰에 참가할 수 있음.