Commodity Description

| Item No. | Product Description | Unit | Q’ty |
| --- | --- | --- | --- |
| 1 | Biomolecular imaging system | Set | 1 |

A. Description

A Biomolecular Imaging System is an advanced device used for visualizing and analyzing biological samples, such as proteins, nucleic acids, cells, and tissues. It typically comprises a light source to detect fluorescence, absorbance, or luminescence signals, a high-resolution camera, filters to improve signal-to-noise ratio, and image analysis software for processing and storing images. Key functions include fluorescence imaging, luminescence imaging, absorbance imaging, high-resolution image capture, and quantitative analysis. These systems are widely used in applications such as protein analysis (e.g., SDS-PAGE, Western Blotting), nucleic acid analysis (e.g., electrophoresis), cell and tissue research, drug development, and molecular interaction studies. The primary benefits of biomolecular imaging systems are their high sensitivity, accuracy, and versatility, making them indispensable tools in life science research for understanding the structure and function of biomolecules.

B. Product specifications

| Dimensions | About Width 500 mm x Height 800 mm x Depth 600 mm |
| --- | --- |
| Weight | 50 kg or less |
| Lens | f/0.95 or higher |
| CCD Detector | 6 megapixels or higher-cooled CCD |
| Automated features | Zoom, focus, exposure, pre-capture and onboard image analysis |
| Illumination Source | Epi-White included and upgradable by including other sources such as UV for a wide variety of applications |
| Application | Colorimetric protein gel, colorimetric membrane imaging, colorimetric western blot imaging, DNA in agarose gel |
| Image Output | TIF or JPG |
| Field of view | 160 × 220 mm or wider |
| Interface | Built-in touch screen |
| Controlled using | External computer |
| Operating Temperature | 28 ℃ or below |

C. Remarks

- Maker’s qualified specialist should carry out the installation and operation.

- Instrument installation qualification (IQ) service

- 2-year-warranty after performance test

- For controlling this device, external PC and its accessories are needed.

D. 위 규격서에서 명시한 제품과 동등 이상의 다른 제품 납품이 가능한 업체는 입찰에 참가할 수 있음.