COMMODITY DESCRIPTION

| HSK NO. | MODEL NO. | DESCRIPTION | UNIT | QUANTITY |
| --- | --- | --- | --- | --- |
|  |  | Centrifuge 1 (Refrigerated Benchtop) | EA | 2 |

A. Features

- Compact, refrigerated multi-purpose bench-top centrifuge.

- Digital displays to indicate actual speed, set speed, temperature and rotor imbalance.

- 10 rotors and a comprehensive section of adaptors allows for a variety of applications, from cell and blood separation to high-throughput screening.

- Its compact footprint frees up space on your bench and its ≤56 dBA (at full speed) makes it a quiet companion in the lab.

- Ability to maintain a consistent temperature, allowing you to spin faster and longer without overheating your samples.

- Rapid Temp and ECO mode allow for even more efficiency and functionality.

- Operation is simple and seamless with soft-touch lid closure, 50 programmable runs and an easy-to-use illuminated interface.

- Biosafety certified aerosol containment bucket covers for 4 x 750mL swinging bucket rotor to prevent escape of biohazardous samples in case of tube/bottle leakage.

- The rotor can be installed, and 100 x 1.5/2 mL microfuge tubes can be used.

- Overspeed system continuously monitors the rotor during centrifugation to ensure that the rotor does not exceed set speed.

- Imbalance detector monitors the rotor during the run causing an automatic shutdown if rotor loads are severely out of balance.

B. Specifications

- Max. RCF: 20,412 * g

- Rotor
1) Swinging-bucket rotor: max. 4*750 mL
2) Fixed-angle rotor: max. 100 * 2 mL

- Swinging Bucket (240V): 4,651 * g

- Plates (240V): 5,495 * g

- Capacity: 4 * 750 mL / 60 * 15 mL / 28 * 50 mL / 4 * 4 plates

- Temperature range: -10 to 40°C

- Acceleration / Deceleration: 10 / 10 sec

- Noise Level: ≤56 dBA at max. speed

- Dimensions(H*W*D): 390*605*635 mm

C. Remarks

- Installation and operation are carried out directly by the supplier at the installation site.

- User manual (English or Korean) is provided.

- 2 years of warranty.