Commodity description: Particle Size Analyzer

| Description | Unit | Quantity |
| --- | --- | --- |
| Particle Size Analyzer | 1 | 1 |

End User Application

- High-resolution measurements and distribution measurements of particles and molecular size from nanometers to several microns.

- Assessing size data, or Zeta potential, of colloids, nanoparticles, polymers, and proteins. Measures the zeta potential of particles and molecules, indicating sample stability and/or propensity to aggregate.

- Measure quickly and with increased repeatability.

- Optical filter wheel with a fluorescence filter and polarizers for analytical flexibility.

- Preferred to have non-invasive back-Scatter technology maximizes the dynamic range, providing the highest sensitivity for the most concentrated samples.

- Able to measure electrophoretic mobility and zeta potential in high-conductivity media

- Software enabled for flexible and guided use, with ease.

Performance and specifications

Particle Size

- Measurements: Size, Zeta Potential, A2, Molecular Weight, Transmission, Refraction

- Detector:  Photodiode Detector

- Temp. Control range: 0°C ~ 120°C

- Ambient operating conditions: 10% to 90% RH, 10°C to 35°C

- Computer Interface: USB 2.0 or higher

- Weight: 10-19 kg

- Dimensions: 300mm x 500mm x 230mm (W x D x H)

Measurement methods

- Dynamic Light Scattering (13°, 173°)

- Multi-angle dynamic light scattering (MADLS)

- Measurement angles: 173°, 13°, 90°

- Measurement range (Diameter): 0.3 nm - 10 μm

- Minimum sample volume: 3μL -40µl

- Minimum sample concentration: 0.1 mg/ml , 1 mg/ml , 5 mg/ml

Zeta Potential measurement

- Measurement technique:  light  scattering

- Zeta potential range: No effective limitations

- Size range suitable for measurement (diameter): 3.0 nm ~ 100 μm

- Mobility range: Minimum zero, no effective maximum

- Minimum sample volume: 20μL -40µL

- Minimum sample concentration: 1 mg/mL -5mg/mL

- Maximum sample conductivity: 10 -260 mS/cm

- Conductivity accuracy: ± 10%

Accessories

- Size measurement cuvettes, Disposable, 100ea/pack – 1 pack

- Disposable Zeta potential measurement cell (Folded capillary cell), 10ea/pack – 1 pack

Operating software

- ZS Xplorer or equivalent software

- IQ/OQ package

PC system

- CPU: 4th Gen+ Intel Core i7 Processor (equivalent or higher)

- Memory: 16 GB RAM

- HDD: 1 TB free hard disk space

- O/S: Windows 10 Professional (64-bit)

- LCD/LED Display Monitor

Remarks

- Warranty: 1-year warranty after installation completion

- A certified engineer should perform the installation and operation training in Korean and English