COMMODITY DESCRIPTION

| HSK NO. | ITEM NO. | DESCRIPTION | UNIT | QUANTITY |
| --- | --- | --- | --- | --- |
|  |  | CO₂ Incubator |  | 3 |

- Features

- Separate Water Reservoir Design:
Designed with a separate (non-integrated) water reservoir, allowing easy refilling and cleaning without disturbing the incubation chamber. This design supports routine operation convenience, reduces contamination risk, and aligns with user-preferred laboratory practices.

- Fast Temperature and CO₂ Recovery:
Rapid recovery of temperature and CO₂ concentration after door opening (typically within 10 minutes), without setpoint overshoot, ensuring stable cell culture conditions.

- Easy-to-Clean Stainless-Steel Chamber:
Seamless stainless-steel interior chamber without fans, ducts, or covers, enabling easy cleaning and minimizing contamination risk.

- High-Temperature Sterilization Capability:
Integrated high-temperature sterilization routine (e.g., up to 180 °C) for effective decontamination of the incubation chamber.

- Comprehensive Data Logging and Documentation:
Capable of recording and documenting CO₂ concentration, temperature, door opening events, alarms, and related parameters to support troubleshooting and quality management.

- GMP / Cleanroom Compatibility:
Designed to be compatible with GMP environments and cleanroom use (e.g., ISO Class 4 / GMP Grade A), supporting regulated laboratory operations.

- Compact and Stackable Design:
Space-efficient footprint with stackable configuration options, suitable for laboratories with limited space.

- Specifications

  - Chamber Volume:
Shall be 165 L or greater, suitable for routine and advanced cell culture applications.

  - External Dimensions (W×D×H):
Shall not exceed approximately 718 × 715 × 900 mm, or equivalent.

  - Product Weight:
Shall be 113 kg or less, or equivalent.

  - Maximum Shelf Load:
Shall support at least 10 kg per shelf.

  - Sterilization Function:
Shall support high-temperature sterilization up to 180 °C.

  - Temperature Control Accuracy:
Shall maintain temperature control within ±0.1 °C.

  - Temperature Uniformity:
Shall provide temperature uniformity within ±0.3 °C.

  - CO₂ Control Range and Accuracy:
Shall support a CO₂ control range of 0.1–20 % with control accuracy of ±0.1 %.

  - Power Supply:
Shall be compatible with 220 V, 50/60 Hz, or equivalent laboratory power supply.

C. Remarks

- The installation and operation should be carried out by the bidder/supplier responsibility at end-user's site.

- IQ and OQ must be included.

- The supplier shall provide a warranty of not less than two (2) years from the date of installation and acceptance.